PEP - Parasitic Extraction Package

by Andrea Casotto and Enrico Malavasi

University of California, Berkeley

Last revised:  Sept 11, 1991

------------------------------------------------------

This is a package of functions for parasitic extraction from oct

symbolic and physical views.

The header file is:	$OCTTOOLS/include/pep.h

The library is:		$OCTTOOLS/lib/libpep.a

The lint library is:	$OCTTOOLS/lib/llib-lpep.ln

Packages Needed: oct, errtrap, tap, th, oh, region

Units: Capacitances are given in Farad, resistances in ohm.

Functions

---------

EXTERN double pepCapacitance

ARGS( (octObject *obj ));

Return the capacitance of the given object (geometry or net) toward

the substrate. Non-hierarchical extraction only is carried out. Net

capacitance is extracted for path implementation only. Terminals can

not be extracted with this function, see pepTermCap.

EXTERN double pepCrossCapacitance

ARGS( (octObject *obj1, octObject *obj2 ));

Return the cross-coupling capacitance between the two objects, which

can be geometries and/or nets. As in pepCapacitance, non-hierarchical

extraction only is carried out.

EXTERN double pepResistance

ARGS( (octObject *source, octObject* target ));

Return the resistance between the two given objects, which can be

geometries and/or terminals. Resistance is computed considering the

net connecting source and target. All the terminals connected to the

net are considered open-circuited. Non-hierarchical extraction

only is performed.

EXTERN double pepPathRes

ARGS( (octObject *path, octObject* t1, octObject* t2 ));

Return the resistance between objects <t1> and <t2> considering only

the given path. Any other eventual path between the two objects is

ignored.

EXTERN double pepTermCap

ARGS((octObject  *term));

Return the capacitance-to-ground associated with the given terminal.

Hierarchy is followed until the physical view is reached.

The lowest layer attached to the terminal is expanded. If it is on an

ACTIVE class layer, it is clipped by any geometry on the GATE-LAYER layer.